Baselining of Value Generation Potential
Blast Furnace Optimization using AI/ML Technology

# Contents

| No. | Section | Page |
| --- | --- | --- |
| 1. | Background & Purpose | 2 |
| 2. | Stakeholder Consultation & Governance | 2 |
| 3. | Blast Furnace Optimization Scope | 2 |
| 4. | Key Methodology & Considerations for Baselining | 3 |
| 5. | Important Notes & Conditions for Re-Baselining | 4 |
| 6. | Baselined KPI & Value Summary | 5 |
| 7. | Approval Request | 6 |
| 8. | Sign-Off & Approval | 6 |

# 1. Background & Purpose

This note is submitted for baselining the anticipated / expected value generation potential from the Blast Furnace Optimization initiative using AI/ML technology.

The project objective is to establish a reference baseline for hot metal quality, operating recipe factors, and controllable blast furnace parameters so that post-implementation performance can be compared objectively against the pre-implementation condition.

Provide early visibility of hot metal temperature and chemistry behavior.

Support operator decision-making using RIST-RAFT logic and data-driven prediction.

Reduce process variability and improve operating consistency.

Create a transparent basis for future value realization and commercial linkage.

# 2. Stakeholder Consultation & Governance

The baseline is intended to be validated with the Blast Furnace operations team, process technology stakeholders, analytics team, and costing/finance representatives before final commercial use.

Blast Furnace operations team to confirm controllable levers and operating constraints.

Process/technical excellence team to validate HMT, chemistry, and recipe interpretation.

Analytics team to validate data preparation, correlation outputs, and model-readiness.

Costing/finance team to validate price parameters and INR value calculation.

# 3. Blast Furnace Optimization Scope

The Blast Furnace Optimization project combines domain logic and predictive analytics. The current scope is centered on hot metal temperature and chemistry stability, recipe-factor relationships, and advisory support for operating levers.

## 3.1 Process Context

Raw materials such as sinter, pellet, ore, coke, and fluxes are charged from the top.

Hot blast, oxygen enrichment, PCI, and moisture influence the lower furnace thermal state.

Outputs include hot metal, slag, top gas, and chemistry indicators such as C, Si, S, P, Mn, Ti, and Cr.

Hot metal temperature stability affects productivity, slag handling, and downstream steelmaking quality.

## 3.2 AI/ML Use Case Intent

Use historical matched cast, recipe, and HM analysis data to establish baseline performance.

Use correlations and model features to identify key process levers influencing hot metal chemistry.

Extend the baseline into HMT prediction and RIST-RAFT based advisory logic as full sensor data becomes available.

# 4. Key Methodology & Considerations for Baselining

## 4.1 Data Matching Logic

The current combined data file matches each cast opening with the latest recipe sent before cast opening, using a six-hour buffer. Hot metal chemistry is mapped at cast level through the cast number.

Input grain: matched cast-level record.

Recipe timing: latest recipe before cast opening.

Output chemistry: HM analysis result by cast number.

Baseline values: average of available matched records for each KPI.

## 4.2 Correlation and Feature Logic

Correlation outputs are used as an explanatory baseline to understand which recipe/process factors have the strongest relationship with hot metal chemistry. These relationships are directional and should be interpreted with process validation, not as final causality.

| HM Element | Top Related Factor | Correlation | Direction |
| --- | --- | --- | --- |
| HM_C | Difference_Minutes | Values to be added | - |
| HM_Cr | CR | Values to be added | - |
| HM_Mn | CAST_NUMBER | Values to be added | - |
| HM_P | SS | Values to be added | - |
| HM_S | S | Values to be added | - |
| HM_Si | NC | Values to be added | - |
| HM_Ti | LS | Values to be added | - |

## 4.3 RIST-RAFT and HMT Prediction Linkage

RIST-RAFT provides first-principles guidance for blast rate and coke rate under given operating conditions.

HMT prediction provides forward-looking temperature visibility so operators can respond before drift becomes visible at tapping.

Controllable levers include PCI, coke rate, blast conditions, oxygen enrichment, moisture, and burden/recipe mix.

# 5. Important Notes & Conditions for Re-Baselining

The current baseline is based on available matched data and should be refreshed using the approved plant data window.

Commercial savings should be calculated only after target values and price assumptions are validated by Costing/Finance.

The same data filters and matching logic should be applied to both baseline and post-implementation periods.

Correlation outputs should be reviewed with process experts before being converted into operator recommendations.

If full time-series sensor data is added, HMT prediction and RIST-RAFT advisory KPIs should be incorporated into the final baseline.

# 6. Baselined KPI & Value Summary

Target values, price parameters, and savings are retained as approval-stage fields to be completed after plant and costing validation.

| Parameter | UOM | Weighted Average Values | Target Values | Optimization Potential | Price Parameter & UoM | Price/Value | Savings in INR Cr/year |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Improvement Parameters |  |  |  |  |  |  |  |
| Hot Metal C |  |  |  |  |  |  |  |
| Hot Metal Si |  |  |  |  |  |  |  |
| Hot Metal S |  |  |  |  |  |  |  |
| Hot Metal Mn |  |  |  |  |  |  |  |
| Hot Metal Ti |  |  |  |  |  |  |  |
| Hot Metal P |  |  |  |  |  |  |  |
| Hot Metal Cr |  |  |  |  |  |  |  |
| Production Parameters |  |  |  |  |  |  |  |
| Sinter |  |  |  |  |  |  |  |
| Pellet |  |  |  |  |  |  |  |
| Ore |  |  |  |  |  |  |  |
| NC |  |  |  |  |  |  |  |
| Dolo |  |  |  |  |  |  |  |
| QTZ |  |  |  |  |  |  |  |
| LS |  |  |  |  |  |  |  |
| SS |  |  |  |  |  |  |  |
| Oxide Wt |  |  |  |  |  |  |  |
| CB |  |  |  |  |  |  |  |
| PCI / Injection Rate |  |  |  |  |  |  |  |
| CR / Coke Rate |  |  |  |  |  |  |  |
| NCR |  |  |  |  |  |  |  |
| Fuel Rate |  |  |  |  |  |  |  |
| Flux Rate |  |  |  |  |  |  |  |
| HM per Charge |  |  |  |  |  |  |  |
| SR |  |  |  |  |  |  |  |
| B2 |  |  |  |  |  |  |  |

Table: Baselined KPIs and expected value realization fields for Blast Furnace Optimization

# 7. Approval Request

In view of the above, approval is requested to use the current matched-data baseline as the working reference for Blast Furnace Optimization using AI/ML technology, subject to final validation of target values and commercial assumptions.

AI/ML baseline and model-readiness assessment

Hot metal chemistry and operating-parameter comparison

Future value realization quantification after target and costing validation

# 8. Sign-Off & Approval

| Role / Department | Name | Signature | Date |
| --- | --- | --- | --- |
| Blast Furnace Operations |  |  |  |
| Process / Technical Excellence |  |  |  |
| Analytics / DCOE |  |  |  |
| Costing / Finance |  |  |  |
| Information Technology |  |  |  |
| Project Owner |  |  |  |